Appendix F: Data and Forecasting Methodology

Introduction

In order to investigate the impact of proposed scenarios for the Cultivating Community effort to increase workforce housing, three different travel demand model (TDM) scenarios were developed. These scenarios used most of the same base assumptions as the 2025 Regional Transportation Plan (RTP), with the primary differences being updated information about development that has taken place since the 2022 base year used for that RTP, and the additional housing and occupancy policies proposed under Alternatives 2 and 3, which are described in detail below. For a detailed description of the assumptions used for the 2025 RTP, please refer to the original memo. Both employment and school enrollment were assumed to increase slightly in Alternatives 2 and 3 in proportion to the increases in population.

Development Scenario Descriptions

Alternative 1 — No Action (Base Year RTP Forecast)

Alternative 1 assumed no change in existing policies and used the same growth assumptions outlined in the 2025 RTP. Minor differences from the Travel Demand Model run for the 2025 RTP were due to starting from a 2025 base year rather than a 2022 base year, which incorporated additional information about the location and number of residential units in the Tahoe Basin and development that occurred between 2022 and 2025. Vacation home rental (VHR) numbers were also adjusted to reflect updated policies in the City of South Lake Tahoe at the time the model was run.

Forecast Methodology

For each alternative, new residential development was allocated spatially to parcels meeting eligibility criteria specific to each unit pool category. Parcels were evaluated for eligibility based on attributes including existing land use, ownership type, IPES score, housing zoning designation, location relative to TRPA Bonus Unit Boundary and Town Centers, and parcel size. Unit allocation within each pool proceeded sequentially until the pool target was reached. Occupancy rates and household income distributions were then applied by pool category along with persons per household at the TAZ level to translate new residential units into occupied households and population estimates for input to the travel demand model.

Alternative 2 — Workforce Housing Focus

Alternative 2 proposes a variety of changes to housing policy designed to increase the number of residential units constructed in the Tahoe Basin. Under this alternative, additional housing would be constructed across several unit pool categories, each with different assumptions about where units would be located. For the purposes of modeling, additional new development was randomly assigned to parcels meeting the eligibility criteria for each pool.

Additional bonus units in this alternative were distributed across the following categories and location eligibility criteria:

| Unit Category | New Units | Location / Eligibility Criteria |
| --- | --- | --- |
| Affordable | 456 | Within designated Town Centers |
| Moderate | 285 | Within the Bonus Unit Boundary |
| Achievable | 228 | Within the Bonus Unit Boundary |
| Achievable | 57 | Anywhere residential development is allowed |
| Affordable by Design | 114 | Anywhere residential development is allowed |
| JADU | 98 | Anywhere residential development is allowed |
| Total Additional Bonus Units | 1,238 |  |

Note: These unit categories are in addition to the Bonus, General, and ADU pools shared across all alternatives. See Table 1 for full unit counts.

Alternative 3 — Increased Usage of Existing Housing

Alternative 3 proposes a variety of changes to housing policy intended to increase the proportion of existing housing in the basin occupied by year-round residents. Under this alternative, new housing development was assigned to parcels in the same manner as Alternative 1 (No Action), and the residential occupancy rate was increased uniformly throughout the basin so that the number of occupied units reached the targeted number of additional occupied residential units. Basin population in this scenario was then calculated by multiplying the occupied residential units by the persons per household at the TAZ level.

Alternative Descriptions and Key Parameters

Table 1: Alternative Descriptions and Key Parameters

| Parameter | Alternative 1 (Baseline) | Alternative 2 (Affordable Housing Focus) | Alternative 3 (Increased Housing Utilization) |
| --- | --- | --- | --- |
| Total New Residential Units | 3,377 | 4,615 | 3,377 |
| Bonus Units | 1,246 | 1,246 | 1,246 |
| General Units | 2,021 | 2,021 | 2,021 |
| ADU | 110 | 110 | 110 |
| Total Additional Units vs. Alternative 1 | — | +1,238 | — |
| Affordable Units | — | 456 | — |
| Moderate Units | — | 285 | — |
| Achievable Bonus | — | 228 | — |
| Achievable General | — | 57 | — |
| Affordable by Design | — | 114 | — |
| JADU | — | 98 | — |
| Occupancy Adjustment (2035) | None | None | +332 occupied units |
| Occupancy Adjustment (2050) | None | None | +855 occupied units |
| Residential Population | 57,611 | 60,244 | 59,472 |